Rectangular lighting profile made of extruded aluminium, with dimensions 386x30mm. The profile consists of 2 flat longitudinal segments from extruded aluminium, with dimensions 150x30mm, with an upside. These profiles have a thickness of 2mm, a weight of 1458 grams/meter and are available in lengths of up to 3 meter in one piece. These are kept together as a unit at the front sides by means of end caps from aluminium.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for pendant installation by means of steel wires or rods.

The profile internally has a number of ribs and slots that improve the rigidity and allow an easy installation of the LED boards. The LED boards for indirect lighting are mounted on a white painted aluminium carrier profile by means of plastic rivets. The LED boards are covered by a semi-spherical satin LED diffuser made of PMMA.

The LED boards are standard equipped with electronic drivers that are mounted on the carrier profiles to the sides of the LED boards for the indirect lighting. These drivers are also available in a DALI/TD dimmable version.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can also be equipped with rotatable or pivotable recessed spot lights of 9W (20° or 42°) in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications.

Dimensions: 386 x 30mm x customized length

Gross luminous flux (LO 325mA/HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (HO 325mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE Fluxcode (indirect) : 03 10 35 06 91

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE