Rectangular lighting profile made of extruded aluminium, with dimensions 65x130mm. This profile has a thickness of 2mm, a weight of 2432 grams/meter and is available in lengths of up to 6 meter in one piece. When choosing optional coupler pieces (L-coupler 90 °, 2-way coupler 45-179 °, T-coupler 2x90 °, 3-way coupler 3x45-270 °, X-coupler 90 ° and 4-way coupler 4x45-225 °) the profiles are mitre cut.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for ceiling-mounted installation, pendant installation via steel wires or rods or wall-mounted installation (direct or via wall brackets).

The profile internally has a number of ribs and slots that improve the rigidity and allow the LED boards to be mounted in the profile by means of a white-painted aluminium bracket. The LED boards can be affixed to the carrier profile by means of plastic rivets. For the indirect lighting the LED boards are also mounted into the profile by means of a white painted carrier profile from aluminium.

The LED boards are standard equipped with electronic drivers that are mounted into the profile under the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom of the profile serves as a light-emitting opening and can be covered by a flush or protruding satin LED diffuser made of PMMA, a microprismatic diffuser (MPO), a VDT louvre from satin aluminium (UGR<19) or a plain infill panel from aluminium. For the indirect lighting, the light-emitting opening is also covered by a flush or protruding  satin LED diffuser made of PMMA.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can also be equipped with 3-circuit tracks, (pivotable) downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications :

Dimensions: 65 x 130mm x customized length (flush led diffuser)

65 x 140mm x customized length (protruding led diffuser on the direct or indirect side)

65 x 150mm x customized length (protruding led diffuser on the direct or indirect side)

Gross luminous flux (LO 325mA/HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) of 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA VDT louvre) : 12,6W/meter

Luminaire power (HO 325mA VDT louvre) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE Fluxcode (flush led diffuser) : 49 80 96 99 70

CIE Fluxcode (protruding led diffuser) : 46 76 93 94 74

CIE Fluxcode (MPO) : 59 88 98 100 62

CIE Fluxcode (VDT louvre) : 80 99 100 100 76

CIE Fluxcode indirect (flush led diffuser) : 26 50 72 1 70

CIE Fluxcode indirect (protruding led diffuser) : 4 20 55 6 74

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE