Circular tube profile for direct light distribution from extruded aluminium, with dimensions Ø100mm. This profile has

a thickness of about 2 mm, a weight of 2054 grams/meter and is available in lengths of up to 6 meter in one piece.

All optional coupler pieces (L-coupler 90 °, 2-way coupler 45-179 °, T-coupler 2x90 °, 3-way coupler 3x45-270 °, X-coupler 90 ° and 4-way coupler 4x45-225 °) are made from the same material as the profile and are painted in an identical colour.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for ceiling-mounted installation, pendant installation via steel wires or rods or wall-mounted installation via wall brackets.

The round profile is divided into two segments.  The upper segment, which is also the biggest (about 2/3 of the diameter),

has a number of ribs and slots that improve the rigidity of the profile. The bottom segment serves as

the light-emitting opening and can also be equipped by a plain infill panel from aluminium, which

Completes the cylinder.

The LED boards can be affixed to the mounting plate by means of plastic rivets. On the backside of the

white-painted mounting plate, the electronic driver and the 3 or 5 pole connection clamps are mounted. These drivers

are also available in a DALI/TD dimmable version. For the indirect lighting, the LED boards are affixed

to the profile by means of plastic rivets.

The light-emitting opening on the direct and indirect is covered by a LED diffuser from satin PMMA.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can also be equipped with 3-circuit tracks, downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications :

Dimensions: Ø100mm x customized length

Gross luminous flux (LO 325mA/HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (HO 325mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE flux code (direct) : 44 74 91 95 86

CIE flux code (indirect) : 00 00 00 00 66

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE