Wall profile for direct, indirect or direct/indirect lighting, with dimensions 82x130mm. The luminaire consists of a U-shaped base made of extruded prismatic polycarbonate with legs of different heights and a slightly curved plain infill panel with a thickness of 2mm and a weight of 835 grams/meter. Both profiles can be delivered in lengths of up to 6 meter in one piece. The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is suited for direct wall-mounted installation. To the backside of the base profile the white painted aluminium carrier is affixed, on which the LED boards are affixed by means of plastic rivets.  The electronic drivers and the connection clams are screwed directly on the base profile. These drivers are also available in a DALI/TD dimmable version. The base has an opening on the backside for the cable entry and several slotted holes for wall-mounting. According to the desired light distribution, the profile is equipped with an aluminium film that distributes the lighting in a direct or indirect way. For direct/indirect lighting, no aluminium film is necessary.

A slightly curved hollow cover profile, made of anodized aluminium or painted in a RAL colour according to polyester powder coating process, is put over the wall profile.  The front sides are closed by 2 end caps, which are affixed to the wall profile and cover plate. The whole luminaire has the shape of a beam from the front.

Standards and specifications.

Dimensions: 82 x 130mm x customized length

Gross luminous flux (LO) : 1 or 2 x 2215 (2700K), 2285 (3000K), 2395 (4000K) or 2500 (TW 2700K-6500K) lumen/meter

Gross luminous flux (HO) : 1 or 2 4430 (2700K), 4570 (3000K), 4785 (4000K) or 5360 (TW 2700K-6500K) lumen/meter

Luminaire power (LO) : 1 or 2 x 14,1W/meter – 23,6W/meter (TW 2700K-6500K)

Luminaire power (HO) : 1 or 2 x 28,3W/meter – 48,3W/meter (TW 2700K-6500K)

CIE flux code (direct or indirect) : 70 93 98 100 71

CIE flux code (direct/indirect) : 64 91 98 42 61

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE