Rectangular lighting profile made of extruded aluminium, with visible dimensions 30x34mm. The profile consists of a base profile with trim and dimensions 50x50mm, a wall thickness of 1,7mm and a weight of 645 gram/meter. It can be delivered in lengths of up to 6 meter in one piece.

When choosing optional coupler pieces (L-coupler 90°, 2-way coupler 45-179°, T-coupler 2x90°, 3-way coupler 3x45-270°, X-coupler 90° and 4-way coupler 4x45-225°) the profiles are mitre cut.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for recessed installation without visible finishing trim in the corner between the ceiling and the wall.

The profile is placed in the provided ceiling opening with the edges of the profile resting on the ceiling and wall. The mounting holes are provided at regular intervals in these edges so that the profile can be screwed to the ceiling and wall. After installation, the edges are finished with plaster and painted together with the ceiling and wall so that only the light opening of 30mm is visible.

The lighting profile shows internally a number of ribs and slots that improve the rigidity. The LED boards are mounted directly into the lighting profile by means of plastic rivets. The LED boards are standard equipped with electronic drivers that are mounted outside the profile in a separate housing with pull relief. These drivers are also available in a DALI/TD dimmable version.

The bottom of the lighting profile serves as the light-emitting opening and can be covered by a protruding or flush satin LED diffuser made of PMMA, a black or white ribbed LED diffuser made of PMMA, a VDT louvre from satin aluminium (UGR<19) or a blind infill panel made of aluminium.

Each profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- or 3-hour autonomy, which are also mounted in a separate housing with pull relief.

The lighting profile can also be equipped with downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The lighting profile has end caps with screws on the ends. The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

The profiles are delivered in anodised aluminium as a standard.

Standards and specifications :

Dimensions: 30 x 34mm x customized length (flush LED diffuser or VDT louvre)

30 x 39mm x customized length (protruding or ribbed LED diffuser)

Gross luminous flux (HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 5360 lumen/meter

Gross luminous flux (VDT louvre at 400mA) : 2715 (2700K), 2785 (3000K), 2930 (4000K) lumen/meter

Gross luminous flux (VDT louvre at 500mA) : 3285 (2700K), 3395 (3000K), 3570 (4000K) lumen/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (HO 325mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 48,3W/meter

Luminaire power : 17,7W/meter (VDT louvre at 400mA) – 22,4W/meter (VDT louvre at 500mA)

CIE Fluxcode (flush satin LED diffuser) : 50 80 96 100 67

CIE Fluxcode (protruding satin LED diffuser) : 46 76 93 100 62

CIE Fluxcode (VDT louvre) : 90 99 100 100 68

CIE Fluxcode (white ribbed LED diffuser) : 50 80 96 100 48

CIE Fluxcode (black ribbed LED diffuser) : 68 90 98 100 9

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE