Rectangular lighting profile made of extruded aluminium, with dimensions 30x90mm.  The luminaire consists of a base profile, with dimensions 30x60mm, a thickness of 1.7mm and a weight of 830 grams/meter and a lighting profile with dimensions 30x30mm, a thickness of 1.5mm and a weight of 650 grams/meter. Both profiles can be delivered in lengths of up to 6 meter in one piece.

When choosing optional coupler pieces (L-coupler 90 °, 2-way coupler 45-179 °, T-coupler 2x90 °, 3-way coupler 3x45-270 °, X-coupler 90 ° and 4-way coupler 4x45-225 °) the profiles are mitre cut.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for ceiling-mounted or pendant installation by means of steel wires or rods.

The base and lighting profiles show internally a number of ribs and slots that improve the rigidity. The lighting profile does not necessarily have the same length as the base profile. The LED boards are mounted directly into the lighting profile by means of plastic rivets. The lighting profile is clicked into the base profile and is equipped with the necessary fall protection.

The LED boards are standard equipped with electronic drivers that are mounted into the base profile under the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom of the lighting profile serves as the light-emitting opening and can be covered by a protruding or flush satin LED diffuser made of PMMA, a black or white ribbed LED diffuser made of PMMA, a VDT louvre from satin aluminium (UGR<19) or a blind infill panel made of aluminium.

The same blind infill panel can also be placed on the base profile in the areas where there is no lighting profile.

The base profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The base and lighting profile can also be equipped with downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The base and lighting profile have end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process. Base profile and lighting profile are available in different RAL colours.

Standards and specifications :

Dimensions: 30 x 90mm x customized length (flush led diffuser or VDT louvre)

30 x 95mm x customized length (protruding or ribbed LED diffuser)

Gross luminous flux (HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 5360 lumen/meter

Gross luminous flux (VDT louvre at 400mA) : 2715 (2700K), 2785 (3000K), 2930 (4000K) lumen/meter

Gross luminous flux (VDT louvre at 500mA) : 3285 (2700K), 3395 (3000K), 3570 (4000K) lumen/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (HO 325mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 48,3W/meter

Luminaire power : 17,7W/meter (VDT louvre at 400mA) – 22,4W/meter (VDT louvre at 500mA)

CIE Fluxcode (flush satin LED diffuser) : 50 80 96 100 67

CIE Fluxcode (protruding satin LED diffuser) : 46 76 93 100 62

CIE Fluxcode (VDT louvre) : 90 99 100 100 68

CIE Fluxcode (white ribbed LED diffuser) : 50 80 96 100 48

CIE Fluxcode (black ribbed LED diffuser) : 68 90 98 100 9

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE