Square lighting profile made of extruded aluminum, with dimensions 85x65mm.  This profile has a thickness of 2mm, a weight of 2091 grams/meter and is available in lengths of up to 6 meter in one piece.

When choosing optional coupler pieces (L-coupler 90 °, 2-way coupler 45-179 °, T-coupler 2x90 °, 3-way coupler 3x45-270 °, X-coupler 90 ° and 4-way coupler 4x45-225 °) the profiles are mitre cut.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The luminaire is designed for ceiling recessed installation by means of supporting brackets.

The profile internally has a number of ribs and slots that improve the rigidity. The LED boards are mounted directly on the deeper bevelled carrier profile by means of plastic rivets. The carrier profile is clicked into the base profile and is equipped with the necessary fall protection.

The LED boards are standard equipped with electronic drivers that are mounted into the profile under the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom of the carrier profile serves as a light-emitting opening and can be covered by an intense 40° lens, a batwing lens, an asymmetric lens and a double asymmetric lens.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can be equipped with 3-circuit tracks. The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process. Base profile and carrier profile are available in different RAL colours.

Standards and specifications :

Dimensions: 85 x 65mm x customized length

Gross luminous flux (HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 5360 (HO) lumen/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (HO 325 mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 48,3W/meter (HO)

CIE flux code (lens intense 40°) : 66 87 96 100 93

CIE flux code (batwing lens) : 43 80 95 100 93

CIE flux code (asymmetric lens) : 59 85 96 100 92

CIE flux code (double asymmetric lens) : 55 87 98 100 92

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE