Rectangular lighting profile made of extruded aluminium, with dimensions 120x30mm.  The luminaire consists of a base profile, with dimensions 60x30mm, a thickness of 1.7mm and a weight of 1030 grams/meter and 2 side profiles with dimensions 30x30mm, a thickness of 1.5mm and a weight of 520 grams/meter. Both profiles can be delivered in lengths of up to 6 meter in one piece.

When choosing optional coupler pieces (L-coupler 90 °, 2-way coupler 45-179 °, T-coupler 2x90 °, 3-way coupler 3x45-270 °, X-coupler 90 ° and 4-way coupler 4x45-225 °) the profiles are mitre cut.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is suited for direct ceiling-mounted installation.

The base and side profiles show internally a number of ribs and slots that improve the rigidity. The side profiles do not necessarily have the same length as the base profile. The LED boards are mounted directly into the base or side profiles by means of plastic rivets. The side profiles are clicked sideways to the base profile.

The LED boards are standard equipped with electronic drivers that are mounted into the base profile. These drivers are also available in a DALI/TD dimmable version.

The bottom of the side profiles serves as a light-emitting opening and can be covered by a satin LED diffuser from PMMA, a VDT louvre from satin aluminium (UGR<19) or a plain infill panel from aluminium. The base profile is also covered by a satin led diffuser from PMMA or a plain infill panel from aluminium.

The base profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

Both the base and side profiles can be equipped with downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The base and side profiles have end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process. Base profile and side profiles are available in different RAL colours.

Standards and specifications :

Dimensions: 120 x 30mm x customized length

Gross luminous flux (LO 325mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (LO 400mA) : 2715 (2700K), 2785 (3000K), 2930 (4000K) lumen/meter

Gross luminous flux (LO 500mA) : 3285 (2700K), 3395 (3000K), 3570 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (LO 400mA) : 17,7W/meter

Luminaire power (LO 500mA) : 22,4W/meter

Luminaire power (HO 325 mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 48,3W/meter (HO)

CIE Fluxcode (satin LED diffuser HO – base profile) : 49 80 96 100 66

CIE Fluxcode (satin LED diffuser HO – side profile) : 50 80 96 100 67

CIE Fluxcode (VDT louvre LO – side profile – direct) : 90 99 100 100 68

CIE Fluxcode (VDT louvre HO – side profile – direct) : 87 99 100 100 55

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE