Square lighting profile made of extruded aluminium, with dimensions 30x30mm.  This luminaire consists of a lighting profile, with dimensions 30x30mm, a thickness of 1.5mm, a weight of 650 grams/meter and is available in lengths of up to 6 meter in one piece.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is suited for recessed installation in SAPP ceilings by means of L-shaped brackets, which can clasp behind the supports of the SAPP ceiling. These supports are equipped with the necessary fixations holes for a fall protection.

The profile internally has a number of ribs and slots that improve the rigidity. The LED boards are mounted directly into the lighting profile by means of plastic rivets.

The LED boards are standard equipped with electronic drivers that are mounted to the exterior of the lighting profile by means of a bracket. These drivers are also available in a DALI/TD dimmable version. These drivers are covered by means of a U-shaped extrusion profile, with the dimensions 30x60mm, which can be clipped on the lighting profile. This profile has a thickness of 2mm and a weight of 830 grams/meter. This profile has the necessary end caps on both sides.

The bottom of the lighting profile serves as the light-emitting opening and can be covered by a satin LED diffuser from PMMA, a VDT louvre from satin aluminium (UGR<19) or a plain infill panel from aluminium. The lighting profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The lighting profile can also be equipped with downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). On both ends, the lighting profile has end caps with visible screws or L-shaped brackets, which can clasp behind the supports of the SAPP ceiling.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions. As a standard, the profiles are delivered with a pre-mounted cable (with pull relief) of 50cm with a 3-pole (black) or 5-pole (blue DALI) wieland connector. Installation has to be carried out according to EN 61535.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications :

Dimensions: 30 x 90mm x customized length

Gross luminous flux (LO 325mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (LO 400mA) : 2715 (2700K), 2785 (3000K), 2930 (4000K) lumen/meter

Gross luminous flux (LO 500mA) : 3285 (2700K), 3395 (3000K), 3570 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (LO 400mA) : 17,7W/meter

Luminaire power (LO 500mA) : 22,4W/meter

Luminaire power (HO 325 mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 48,3W/meter (HO)

CIE Fluxcode (satin LED diffuser HO) : 50 80 96 100 67

CIE Fluxcode (VDT louvre LO) : 90 99 100 100 68

CIE Fluxcode (VDT louvre HO) : 87 99 100 100 55

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE