Rectangular lighting profile made of extruded aluminium, with dimensions 48x70mm. This profile has a thickness of 2mm, a weight of 1154 grams/meter and is available in lengths of up to 6 meter in one piece. When choosing optional coupler pieces (L-coupler 90°, 2-way coupler 45-179°, T-coupler 2x90°, 3-way coupler 3x45-270°, X-coupler 90° and 4-way coupler 4x45-225°) the profiles are mitre cut.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for recessed installation with a visible border via mounting brackets.

The profile internally has a number of ribs and slots that improve the rigidity and allow the LED boards to be mounted in the profile by means of an aluminium mounting plate painted in white. The LED boards are affixed to the mounting plate with plastic rivets.

The LED boards are standard equipped with electronic drivers that are mounted into the profile under the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom of the profile serves as the light-emitting opening and can be covered by a flush or protruding satin LED diffuser made of PMMA, a microprismatic diffuser (MPO), a VDT louvre from satin aluminium (UGR<19) or a plain infill panel from aluminium.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can also be equipped with 3-circuit tracks, (pivotable) downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications :

Dimensions: 48 x 70mm x customized length (flush led diffuser)

48 x 80mm x customized length (protruding led diffuser)

Gross luminous flux (LO 325mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (LO 400mA) : 2715 (2700K), 2785 (3000K), 2930 (4000K) lumen/meter

Gross luminous flux (LO 500mA) : 3285 (2700K), 3395 (3000K), 3570 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (LO 400mA) : 17,7W/meter

Luminaire power (LO 500mA) : 22,4W/meter

Luminaire power (HO 325 mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) or 48,3W/meter (HO)

CIE Fluxcode (flush LED diffuser) : 49 80 96 100 66

CIE Fluxcode (protruding LED diffuser) : 45 75 92 94 65

CIE Fluxcode (MPO) : 58 89 98 100 50

CIE Fluxcode (VDT louvre LO) : 89 100 100 100 68

CIE Fluxcode (VDT louvre HO) : 86 100 100 100  61

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K en 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE