Rectangular lighting profile made of extruded aluminium, with dimensions 170x50mm. The luminaire consists of a base profile, with a thickness of 2mm and a weight of 2112 grams/meter and 2 side profiles (U-shaped) with a thickness of 2mm and a weight of 638 grams/meter, which can be clicked on the base profile. Both profiles can be delivered in lengths of up to 6 meter in one piece.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for pendant installation by means of steel wires or rods.

The profile internally has a number of ribs and slots that improve the rigidity and allow an easy installation of the LED boards. The LED boards are mounted directly on the white painted base profile by means of plastic rivets. The LED boards are standard equipped with electronic drivers that are mounted into the base profile to the sides of the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom serves as the light-emitting opening and can be covered by a satin LED diffuser made of PMMA or a plain infill panel from aluminium. For the indirect lighting, the light-emitting opening is covered by a satin LED diffuser made of PMMA.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can also be equipped with rotatable or pivotable recessed spot lights of 9W (20° or 42°) in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications.

Dimensions: 170 x 50mm x customized length

Gross luminous flux (direct HO 225mA) : 4600 (3000K), 4810 (4000K) lumen/meter

Gross luminous flux (indirect LO 325mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (indirect HO 325mA) : 4430 (2700K), 4570 (3000K), 4785  (4000K) lumen/meter

Gross luminous flux (indirect Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (direct) : 29,6W/meter

Luminaire power (indirect LO) : 14,1W/meter

Luminaire power (indirect HO) : 28,3W/meter

Luminaire power (indirect TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE flux code (direct) : 49 80 96 99 70

CIE flux code (indirect) : 00 00 00 00 73

Lifespan LED module (29,6W): L80/B10 >60000 hours

Lifespan LED module (14,1W and 28,3W) : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE