Rectangular lighting profile with rounded sides made of extruded aluminium, with dimensions 144x40mm. This profile has a thickness of 2mm, a weight of 2106 grams/meter and is available in lengths of up to 6 meter in one piece.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for ceiling-mounted or pendant installation by means of steel wires or rods.

The profile internally has a number of ribs and slots that improve the rigidity and allow an easy installation of the LED boards. The LED boards are mounted directly on the profile by means of plastic rivets. The LED boards are standard equipped with electronic drivers that are mounted into the profile to the sides of the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom serves as the light-emitting opening and can be covered by a satin LED diffuser made of PMMA, a microprismatic diffuser (MPO), a VDT louvre from satin aluminium (UGR<19) or a plain infill panel from aluminium.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can also be equipped with (pivotable) downlight modules of 3x1W or 4x1W in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications.

Dimensions: 144 x 40mm x customized length

Gross luminous flux (LO 325mA/HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA VDT louvre) : 12,6W/meter

Luminaire power (HO 325mA VDT louvre) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE Fluxcode (satin LED diffuser) : 49 80 96 99 67

CIE Fluxcode (MPO) : 58 89 98 100 54

CIE Fluxcode (VDT louvre) : 80 99 100 100 71

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE