Lighting profile made of extruded aluminium, with dimensions 100x110mm. The base profile has the shape of a rectangle with slanting legs at the bottom. This profile has a thickness of 2mm, a weight of 2140 grams/meter and is available in lengths of up to 6 meter in one piece.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for ceiling mounting, suspended mounting by means of steel wires or rods and trimless (semi-)recessed installation by means of a finishing profile.

The profile internally has a number of ribs and slots that improve the rigidity and allow the LED boards to be mounted in the profile by means of an aluminium bracket. The LED boards can be affixed to the carrier profile by means of plastic rivets. For the indirect lighting, the LED boards are put into the profile by means of plastic rivets.

The LED boards are standard equipped with electronic drivers that are mounted into the profile under the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom serves as the light-emitting opening and can be covered by a curved satin LED diffuser made of PMMA. For the indirect lighting, the light-emitting opening is covered by a satin LED diffuser made of PMMA.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process.

Standards and specifications :

Dimensions: 100 x 110mm x customized length

Gross luminous flux (LO 325mA/HO 150mA-indirect) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (LO 325mA-indirect) : 14,1W/meter

Luminaire power (HO 150mA-indirect) : 12,6W/meter

Luminaire power (HO 325mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE Fluxcode (curved satin LED diffuser) : 54 85 98 99 72

CIE Fluxcode indirect (satin LED diffuser) : 00 00 00 00 73

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE