Rectangular lighting profile made of extruded aluminium, with dimensions 200x50mm. The luminaire consists of a U-shaped base profile(150x50mm), has a thickness of 2mm and a weight of 2179 grams/meter and a U-shaped side profile (50x50mm) and a thickness of 2mm and a weight of 1300 grams/meter. Both profiles are linked together by means of springs and are delivered in lengths of up to 6 meter in one piece.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for ceiling-mounted or pendant installation by means of steel wires or rods.

The profile internally has a number of ribs and slots that improve the rigidity and allow an easy installation of the LED boards. The LED boards are mounted in the base profile and side profile onto an aluminium bracket by means of plastic rivets. The LED boards are standard equipped with electronic drivers that are mounted into the base profile to the sides of the LED boards. These drivers are also available in a DALI/TD dimmable version.

The bottom and top of both the base and side profiles (depending on how the profiles are linked together) serve as a light-emitting opening and can be covered by a satin LED diffuser made of PMMA, a microprismatic diffuser (MPO) or a plain infill panel made of aluminium.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The profile can also be equipped with 3-circuit tracks (side profile), rotatable or pivotable recessed spot lights of 9W (20° or 42°) in 2700, 3000 or 4000 Kelvin, motion detectors and/or daylight detectors and push switches (on/off and dimming). The profile has end caps without visible screws at both ends.

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process. Base profile and side profile are available in different RAL colours.

Standards and specifications.

Dimensions: 200 x 50mm x customized length

Gross luminous flux (base profile HO 225mA) : 4600 (3000K), 4810 (4000K) lumen/meter

Gross luminous flux (side profile–LO 325mA/HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (side profile-HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (base profile HO 225mA) : 29,6W/meter

Luminaire power (side profile-LO 325mA) : 14,1W/meter

Luminaire power (side profile-HO 150mA) : 12,6W/meter

Luminaire power (side profile-HO 325mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE Fluxcode (base profile) : 49 80 96 99 65 (direct) / 00 00 00 00 65 (indirect)

CIE Fluxcode (side profile – satin led diffuser) : 49 80 96 100 66 (direct) / 00 00 00 00 65 (indirect)

CIE Fluxcode (side profile – MPO) : 59 89 98 100 50

CIE Fluxcode (side profile – VDT louvre LO) : 89 100 100 100 68

CIE Fluxcode (side profile – VDT louvre HO) : 86 100 100 100 61

Lifespan LED module (29,6W) : L80/B10 >60000 hours

Lifespan LED module (14,1W and 28,3W) : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I

Protection rating: IP 20

Certificates: ISO 9001 - ENEC - CE