Beam-shaped lighting profile made of extruded aluminium, with dimensions 70x85mm. The profile is beam-shaped made of satin plexi. The base profile has a thickness of 2mm, a weight of 746 grams/meter and is available in lengths of up to 6 meter in one piece.

When choosing optional coupler pieces (L-coupler 90 °, 2-way coupler 45-179 °) the profiles are mitre cut.

The luminaire is suitable for LED boards in 2700, 3000, 4000 Kelvin or Tunable White 2700-6500 Kelvin, which are Zhaga-conform (book 7).

The profile is designed for ceiling-mounted or pendant installation by means of steel wires or rods.

The lighting profile consists of a base profile from extruded and white painted aluminium to which a U-shaped mounting plate can be affixed with springs.  The LED boards can be affixed to the mounting plate by means of plastic rivets. A semi-globular satin LED diffuser is placed over the LED boards to enable a homogeneous distribution of the direct and indirect lighting. On the backside of this likewise white painted mounting plate, the electronic driver and the 3 or 5 pole connection clamps are mounted. These drivers are also available in a DALI/TD dimmable version. Over the base profile a U-shaped extruded plexi from satin PMMA is inserted. The front sides are equipped with 2 end caps from anodized aluminium, which are screwed to the base profile in a deeper position. Between light-emitting zones, the base profile is covered by a recessed U-shaped plain infill panel from aluminium. The ends caps have visible screws.

Every profile can optionally be equipped with self-testing (DALI) emergency modules with a 1- to 3-hour autonomy.

The plain infill panel can be equipped with motion detectors and/or daylight detectors and push switches (on/off and dimming).

The lighting system is fully pre-assembled, pre-wired and made exactly as agreed when it is delivered on site, together with its mounting instructions.

As a standard, the profiles are delivered in anodised aluminium or in a RAL colour of choice, according to the polyester powder coating process. Base profile and edge profile are available in different RAL colours.

Standards and specifications :

Dimensions: 70 x 85mm x customized length

Gross luminous flux (LO 325mA/HO 150mA) : 2215 (2700K), 2285 (3000K), 2395 (4000K) lumen/meter

Gross luminous flux (HO 325mA) : 4430 (2700K), 4570 (3000K), 4785 (4000K) lumen/meter

Gross luminous flux (Tunable White 2700-6500K) : 2500 (LO) or 5360 (HO) lumen/meter

Luminaire power (LO 325mA) : 14,1W/meter

Luminaire power (HO 150mA) : 12,6W/meter

Luminaire power (HO 325mA) : 28,3W/meter

Luminaire power (TW 2700K-6500K) : 23,6W/meter (LO) – 48,3W/meter (HO)

CIE flux code (satin LED diffuser) : 34 61 83 76 75

Lifespan LED module : L80/B10 >72000 hours

Lifespan LED module Tunable White : L80/B10 >50000 hours

CRI : 80 (2700K, 3000K and 4000K) – 90 (TW 2700K-6500K)

Colour tolerance : MacAdam 3

Class: I